Aim:- Packet Tracer - Configure AAA Authentication on Cisco Routers

Part 1: Configure Local AAA Authentication for Console Access on R1

Step 1: Test connectivity.

• Ping from PC-A to PC-B.

• Ping from PC-A to PC-C.

• Ping from PC-B to PC-C.

Step 2: Configure a local username on R1.

Configure a username of Admin1 with a secret password of admin1pa55.

R1(config)# username Admin1 secret admin1pa55

Step 3: Configure local AAA authentication for console access on R1.

Enable AAA on R1 and configure AAA authentication for the console login to use the local database.

R1(config)# aaa new-model

R1(config)# aaa authentication login default local

Step 4: Configure the line console to use the defined AAA authentication method.

Enable AAA on R1 and configure AAA authentication for the console login to use the default method list.

R1(config)# line console 0

R1(config-line)# login authentication default

Step 5: Verify the AAA authentication method.

Verify the user EXEC login using the local database.

R1(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R1# exit

R1 con0 is now available Press

RETURN to get started.

************ AUTHORIZED ACCESS ONLY *************

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

User Access Verification

Username: Admin1

Password: admin1pa55

R1>

Part 2: Configure Local AAA Authentication for vty Lines on R1

Step 1: Configure domain name and crypto key for use with SSH.

a. Use ccnasecurity.com as the domain name on R1.

R1(config)# ip domain-name ccnasecurity.com

b. Create an RSA crypto key using 1024 bits.

R1(config)# crypto key generate rsa

Choose the size of the key modulus in the range of 360 to 2048 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK] Step

2: Configure a named list AAA authentication method for the vty lines on R1.

Configure a named list called SSH-LOGIN to authenticate logins using local AAA.

R1(config)# aaa authentication login SSH-LOGIN local

Step 3: Configure the vty lines to use the defined AAA authentication method.

Configure the vty lines to use the named AAA method and only allow SSH for remote access.

R1(config)# line vty 0 4

R1(config-line)# login authentication SSH-LOGIN

R1(config-line)# transport input ssh R1(config-line)#

End

Step 4: Verify the AAA authentication method.

Verify the SSH configuration SSH to R1 from the command prompt of PC-A..

PC> ssh –l Admin1 192.168.1.1

Open

Password: admin1pa55

Part 3: Configure Server-Based AAA Authentication Using TACACS+ on R2

Step 1: Configure a backup local database entry called Admin.

For backup purposes, configure a local username of Admin2 and a secret password of admin2pa55.

R2(config)# username Admin2 secret admin2pa55

Step 2: Verify the TACACS+ Server configuration.

Click the TACACS+ Server. On the Services tab, click AAA. Notice that there is a Network configuration

entry for R2 and a User Setup entry for Admin2.

Step 3: Configure the TACACS+ server specifics on R2.

Configure the AAA TACACS server IP address and secret key on R2.

Note: The commands tacacs-server host and tacacs-server key are deprecated. Currently, Packet Tracer

does not support the new command tacacs server.

R2(config)# tacacs-server host 192.168.2.2 R2(config)#

tacacs-server key tacacspa55

Step 4: Configure AAA login authentication for console access on R2.

Enable AAA on R2 and configure all logins to authenticate using the AAA TACACS+ server. If it is not

available, then use the local database.

R2(config)# aaa new-model

R2(config)# aaa authentication login default group tacacs+ local

Step 5: Configure the line console to use the defined AAA authentication method.

Configure AAA authentication for console login to use the default AAA authentication method.

R2(config)# line console 0

R2(config-line)# login authentication default

Step 6: Verify the AAA authentication method.

Verify the user EXEC login using the AAA TACACS+ server.

R2(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R2# exit

R2 con0 is now available Press

RETURN to get started.

************ AUTHORIZED ACCESS ONLY *************

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

User Access Verification

Username: Admin2

Password: admin2pa55

R2>

Part 4: Configure Server-Based AAA Authentication Using RADIUS on R3

Step 1: Configure a backup local database entry called Admin.

For backup purposes, configure a local username of Admin3 and a secret password of admin3pa55.

R3(config)# username Admin3 secret admin3pa55

Step 2: Verify the RADIUS Server configuration.

Click the RADIUS Server. On the Services tab, click AAA. Notice that there is a Network configuration entry

for R3 and a User Setup entry for Admin3.

Step 3: Configure the RADIUS server specifics on R3.

Configure the AAA RADIUS server IP address and secret key on R3.

Note: The commands radius-server host and radius-server key are deprecated. Currently Packet Tracer

does not support the new command radius server.

R3(config)# radius-server host 192.168.3.2 R3(config)#

radius-server key radiuspa55

Step 4: Configure AAA login authentication for console access on R3.

Enable AAA on R3 and configure all logins to authenticate using the AAA RADIUS server. If it is not available,

then use the local database.

R3(config)# aaa new-model

R3(config)# aaa authentication login default group radius local

Step 5: Configure the line console to use the defined AAA authentication method.

Configure AAA authentication for console login to use the default AAA authentication method.

R3(config)# line console 0

R3(config-line)# login authentication default

Step 6: Verify the AAA authentication method.

Verify the user EXEC login using the AAA RADIUS server.

R3(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R3# exit

R3 con0 is now available Press

RETURN to get started.

************ AUTHORIZED ACCESS ONLY *************

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

User Access Verification

Username: Admin3

Password: admin3pa55

R3>

OUTPUT: